Test Date: May 29, 2024

Jack Parent | jack.parent@transpak.com

Annie Schaubel | annie.schaubel@transpak.com

Camera Pack

ISTA 2A-2011

Rev A

TransPak

20415 Corsair Blvd.

Hayward, CA 94545

# Table of Contents

Table of Contents	2

Objective	3

Key Takeaways	3

Test Protocol	4

Test Equipment	5

Test Sample Description	6

Pre-test Photos	6

Test Sequence 1 – Ambient Atmospheric Preconditioning	8

Test Sequence 2 – Controlled Atmospheric Preconditioning	8

Test Sequence 3 – Compression	8

Test Sequence 4 – Random Vibration	10

Test Sequence 5 – Shock	12

Test Sequence 6 – Random Vibration	14

Post-Test Photos	16

Appendix A – Test Sequence 4 Vibration Data	17

Appendix B – Test Sequence 6 Vibration Data	21

Certification

TransPak is an ISTA certified testing facility – Member ID: 10149 International Safe Transit Association 1400 Abbott Road, Suite 160 East Lansing, MI 48823- 1900 Email: ista@ista.org

# Objective

To validate the ability of the packaging and product to withstand shipping environment hazards utilizing ISTA 2A-2011, Partial Simulation Performance Test Procedure, Packaged-Products 150 lb or Less. Using one (1) package system, TransPak performed a series of tests that simulate hazards which may be present in a standard shipping environment.

# Key Takeaways

- One (1) Camera Pack package system provided by Cavnue was tested using ISTA 2A-2011 test procedure.

- No visible critical damage to the packaging was observed upon pre-test inspection.

- Product inspection required by the customer to determine if any damage to the product has occurred, whether functional or cosmetic.

# Test Protocol

| Sequence No. | Test Category | Test Type | Test Level |
| --- | --- | --- | --- |
| 1 | Atmospheric Preconditioning | Temperature and Humidity | Ambient |
| 2 | Atmospheric Preconditioning | Controlled Temperature and Humidity | 72 hours at Controlled Conditions - 23°C and 50% RH +/- 5% |
| 3 | Compression | Machine Apply and Release | 1,739.5 lbs |
| 4 | Vibration | Random | 1 hour at 1.15 Grms |
| 5 | Shock | Drop | 10 drops at 32” |
| 6 | Vibration | Random | 1 hour at 1.15 Grms |

# Test Equipment

| Equipment Used |  |  |  |  |
| --- | --- | --- | --- | --- |
| Equipment | Make | Model | S/N | Calibration Due Date |
| Swingarm Drop Tester | Lansmont | ODT-56ED | M-11233 | 03/28/2025 |
| Vibration Test System | Lansmont | 10000 | 18163 | 03/28/2025 |
| Vibration Controller | Lansmont | TTV4 | 74538 | 03/28/2025 |
| Compression Test Machine | Haida International Equipment Co., Ltd. | HD-A502S-1500 | N/A | N/A |
| Triaxial Accelerometer | PCB Piezotronics | 356A24 | 346135 | 5/08/2025 |

# Test Sample Description

| Packaged-Product Materials | Total Weight | Outside Dimensions (L x W x H) | Quantity |
| --- | --- | --- | --- |
| 350# BC-flute double-wall Kraft corrugated RSC | 24.85 lbs | 24” x 19.75” x 18” | 1 |
| 1.7# PE Laminated Foam - Black |  | N/A | 1 |
| Customer Product Sample |  | N/A | 1 |

# Pre-test Photos

# Test Sequence 1 – Ambient Atmospheric Preconditioning

The package system was exposed to ambient atmospheric conditions for a minimum duration of twelve (12) hours.

# Test Sequence 2 – Controlled Atmospheric Preconditioning

The package system was conditioned to the appropriate Anticipated Conditions as defined in ISTA 2A-2011, page 6. The “Controlled Conditions” profile was selected as this profile is the most similar to typical conditions experienced in Northern California.

| Anticipated Conditions | Time in Hours | Temperature in °C +/- 2°C | Humidity in % |
| --- | --- | --- | --- |
| Controlled Conditions | 72 | 23°C | 50% RH +/- 5% |

# Test Sequence 3 – Compression

Immediately following Test Sequence 2, Compression (Machine Apply and Release) was performed according to the defined test procedure in ISTA 2A-2011. A compression force of 1,739.5 lbs was determined using the following equation:

[Wt×(S-1)×F]×1.4

Wt – Total weight of the packaged-product – 24.85 lbs

S – Total number of packaged-products in a stack – 11 (calculated using guidance provided in ISTA 2A)

F – Compensating factor – 5 (ISTA recommends a minimum of 5 if the packaged-product may be warehoused during distribution)

The compression test procedure was performed using a compression testing machine; see report below of results:

| Conclusion | Pass / Fail |
| --- | --- |
| Packaged-product achieved the calculated compression force. | Pass |

# Test Sequence 4 – Random Vibration

Random vibration was performed for 1 hour following Test Sequence 3 - Compression. See table below:

| Random Vibration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test No. | Orientation | Vibration Profile | Vibration Intensity (Grms) | Duration (min) | Resonant Frequency (Hz) | Transmissibility (Q) |
| 1 | Face 3 | ISTA 2A-2011 | 1.15 | 30 | 14.75 | 3.08 |
| 2 | Face 1 |  |  | 10 | 13.75 | 2.88 |
| 3 | Face 4 |  |  | 10 | 11.5 | 3.06 |
| 4 | Face 6 |  |  | 10 | 13.5 | 3.12 |
| Graph data of frequency response and transmissibility is located in Appendix A |  |  |  |  |  |  |

The ISTA 2A-2011 acceleration versus frequency vibration profile spectrum is shown below:

| Face 3 | Face 1 |
| --- | --- |
| Face 4 | Face 6 |

| Conclusion | Pass / Fail |
| --- | --- |
| No critical damage or conditions were observed post random vibration testing. | Pass |

# Test Sequence 5 – Shock

Packaged-product was subjected to 10 free fall drops at 32”. Drop height was determined according to the following chart provided in ISTA 2A-2011, Page 9 below:

| Packaged-Product Weight (pounds) |  | Drop Height (inches) |
| --- | --- | --- |
| Equal to or greater than | But Less than | Free Fall |
| 0 | 21 | 38 |
| 21 | 41 | 32 |
| 41 | 61 | 26 |
| 61 | 100 | 20 |
| 100 | 150 | 12 |

The drop sequence was performed according to the table below:

| Drop # | Drop Height (in.) | Face | Peak G’s (axis) |
| --- | --- | --- | --- |
| 1 | 32 | Corner 2-3-5 | 13.48 (X) |
| 2 |  | Edge 2-5 | 17.25 (Y) |
| 3 |  | Edge 3-5 | 23.13 (X) |
| 4 |  | Edge 2-3 | 15.57 (Y) |
| 5 |  | Face 6 | 26.06 (X) |
| 6 |  | Face 5 | 37.78 (X) |
| 7 |  | Face 2 | 39.69 (Y) |
| 8 |  | Face 4 | 28.19 (Y) |
| 9 |  | Face 3 | 38.94 (Z) |
| 10 |  | Face 1 | 24.23 (Z) |
| Waveform data available upon request. |  |  |  |

32” Drop Height

| Conclusion | Pass / Fail |
| --- | --- |
| No critical damage or conditions were observed post free fall drop testing. | Pass |

# Test Sequence 6 – Random Vibration

Random vibration was performed for 1 hour following Test Sequence 5 - Shock. See table below:

| Random Vibration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test No. | Orientation | Vibration Profile | Vibration Intensity (Grms) | Duration (min) | Resonant Frequency (Hz) | Transmissibility (Q) |
| 1 | Face 3 | ISTA 2A-2011 | 1.15 | 30 | 15.5 | 2.65 |
| 2 | Face 1 |  |  | 10 | 10 | 2.98 |
| 3 | Face 4 |  |  | 10 | 10.5 | 2.96 |
| 4 | Face 6 |  |  | 10 | 11.75 | 3.13 |
| Graph data of frequency response and transmissibility is located in Appendix B |  |  |  |  |  |  |

The ISTA 2A-2011 acceleration versus frequency vibration profile spectrum is shown below:

| Face 3 | Face 1 |
| --- | --- |
| Face 4 | Face 6 |

| Conclusion | Pass / Fail |
| --- | --- |
| No critical damage or conditions were observed post random vibration testing. | Pass |

# Post-Test Photos

# Appendix A – Test Sequence 4 Vibration Data

Face 3 Transmissibility Report

Face 1 Transmissibility Report

Face 4 Transmissibility Report

Face 6 Transmissibility Report

# Appendix B – Test Sequence 6 Vibration Data

Face 3 Transmissibility Report

Face 1 Transmissibility Report

Face 4 Transmissibility Report

Face 6 Transmissibility Report

End of Report