Test Date: August 2, 2023

Test Operator #1 Name | Email

Test Operator #2 Name | Email

Product

Modified ISTA 3A Test Protocol

Rev A

TransPak

20415 Corsair Blvd.

Hayward, CA 94545

# Table of Contents

Table of Contents	2

Objective	3

Key Takeaways	3

Test Protocol	4

Test Equipment	5

Test Sample Description	6

Pre-test Photos	6

Test Sequence 1	8

Test Sequence 2	8

Test Sequence 3	9

Test Sequence 4	16

Post-Test Photos	17

Certification

TransPak is an ISTA certified testing facility – Member ID: 10149 International Safe Transit Association 1400 Abbott Road, Suite 160 East Lansing, MI 48823- 1900 Email: ista@ista.org

# Objective

To validate the ability of the packaging and product to withstand shipping environment hazards utilizing a Modified ISTA 3A-2018 General Simulation Performance Test Procedure. Using one (1) package system, TransPak performed a series of tests that simulate hazards which may be present in a standard shipping environment.

# Key Takeaways

- One (1) Product Name package system provided by Company Name was tested using a modified ISTA 3A test procedure (no top load vibration).

- No visible critical damage to the packaging was observed upon pre-test inspection.

- Internal inspection required by the customer to determine if any damage to the product has occurred.

# Test Protocol

| Sequence No. | Test Category | Test Type | Test Level |
| --- | --- | --- | --- |
| 1 | Atmospheric | Preconditioning | Ambient lab conditions for >12 hours |
| 2 | Shock | Free-Fall Drop | 9 Drops |
| 3 | Vibration | Random (without topload) | Overall Grms levels of 0.53 and 0.46 |
| 4 | Shock | Free-Fall Drop | 8 Drops |

# Test Equipment

| Equipment Used |  |  |  |  |
| --- | --- | --- | --- | --- |
| Equipment | Make | Model | S/N | Calibration Due Date |
| Swingarm Drop Tester | Lansmont | ODT-56ED | M-11233 | 03/28/2025 |
| Vibration Test System | Lansmont | 10000 | 18163 | 03/28/2025 |
| Vibration Controller | Lansmont | TTV4 | 74538 | 03/28/2025 |
| Triaxial Accelerometer | PCB | 356A24 | 241279 | 5/08/2025 |

# Test Sample Description

| External Packaging | Weight | Outside Dimensions (L x W x H) | Quantity |
| --- | --- | --- | --- |
| 3mm Wgile | 4.1 lbs | 21.75 X 21 X 7.375 | 1 |

# Pre-test Photos

# Test Sequence 1

The package system was exposed to ambient laboratory conditions for a minimum duration of twelve (12) hours.

# Test Sequence 2

Free Fall Drop (First Sequence)

| Drop # | Drop Height (in.) | Face | Peak G’s |
| --- | --- | --- | --- |
| 1 | 18 | Edge 3-4 |  |
| 2 |  | Edge 3-6 |  |
| 3 |  | Edge 4-6 |  |
| 4 |  | Corner 3-4-6 |  |
| 5 |  | Corner 2-3-5 |  |
| 6 |  | Edge 2-3 |  |
| 7 |  | Edge 1-2 |  |
| 8 | 36 | Face 3 |  |
| 9 | 18 | Face 3 |  |
| Waveform data available upon request. |  |  |  |

| 18” Drop Height | 36” Drop Height |
| --- | --- |

| Conclusion | Pass / Fail |
| --- | --- |
| No critical damage or conditions were observed post free fall drop testing. | Pass |

# Test Sequence 3

| Random Vibration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test No. | Orientation | Vibration Profile | Vibration Intensity (Grms) | Duration (min) | Resonant Frequency (Hz) | Transmissibility (Q) |
| 1 | Face 3 | Over the Road Spectrum | 0.53 | 60 |  |  |
| 2 | Face 4 |  |  | 30 |  |  |
| 3 | Face 6 |  |  | 30 |  |  |
| 4 | Face 3 | Pick Up and Delivery Vehicle Spectrum | 0.46 | 30 |  |  |
| No topload was applied during testing. |  |  |  |  |  |  |

Face 3 Over-The-Road Spectrum Transmissibility Report

Face 4 Over-The-Road Spectrum Transmissibility Report

Face 6 Over-The-Road Spectrum Transmissibility Report

Face 3 Pick Up & Delivery Transmissibility Report

| Image | Image |
| --- | --- |

| Conclusion | Pass / Fail |
| --- | --- |
| No critical damage or conditions were observed post random vibration testing. | Pass |

# Test Sequence 4

Free Fall Drop (Second Sequence)

| Drop # | Drop Height (in.) | Face | Peak G’s |
| --- | --- | --- | --- |
| 10 | 18 | Edge 3-4 |  |
| 11 |  | Edge 3-6 |  |
| 12 |  | Edge 1-5 |  |
| 13 |  | Corner 3-4-6 |  |
| 14 |  | Corner 1-2-6 |  |
| 15 |  | Corner 1-4-5 |  |
| 16 | 36 | Most critical or damage-prone flat orientation Face 3 |  |
| 17 | 18 | Face 3 on hazard |  |
| Waveform data available upon request. |  |  |  |

| Drop #16 | Drop #17 |
| --- | --- |

| Conclusion | Pass / Fail |
| --- | --- |
| No critical damage or conditions were observed post free fall drop testing. | Pass |

# Post-Test Photos

End of Report